Alkalinity Reference & Formula Blueprint

A Comprehensive Technical Guide for DIY Ratiometric Reagentless Monitoring

# 1. The Master Equation

The ratiometric model processes data across a single pH and temperature probe over a closed-loop reference sample, mathematically cancelling out room-level CO2 fluctuations without the use of chemical reagents or liquid titration acids:

TA_tank = (TA_ref - BA_ref) * [ Carbonate Speciation Ratio ] + BA_tank

Where Carbonate Speciation Ratio =
((K1_tank / [H+]_tank) + (2 * K1_tank * K2_tank / [H+]_tank^2))  /  ((K1_ref / [H+]_ref) + (2 * K1_ref * K2_ref / [H+]_ref^2))

# 2. Comprehensive Parameter Lexicon

| Symbol / Variable | Technical Nomenclature | Plain-English Operational Function |
| --- | --- | --- |
| TA_tank | Total Alkalinity of the Tank | The unknown, real-time buffering capacity of your aquarium water that the device is running calculations to solve for. |
| TA_ref | Total Alkalinity of the Reference | The absolute hardcoded anchor value of your isolated reference reservoir water, measured exactly once on Day 1 using a manual test kit. |
| [H+]_tank | Hydrogen Ion Activity (Tank) | The live aquarium water sample's pH reading converted to an active raw chemical molar concentration via the formula: 10^(-pH_tank). |
| [H+]_ref | Hydrogen Ion Activity (Reference) | The isolated reference water sample's pH reading converted to an active raw chemical molar concentration via the formula: 10^(-pH_ref). |
| K1_tank / K1_ref | First Dissociation Constant of Carbonic Acid | A temperature-dependent thermodynamic constant tracking how easily carbon dioxide converts into single-charged Bicarbonate (HCO3-) ions. |
| K2_tank / K2_ref | Second Dissociation Constant of Carbonic Acid | A temperature-dependent thermodynamic constant tracking how easily Bicarbonate converts into double-charged Carbonate (CO3^2-) ions. |
| BA_tank / BA_ref | Borate Alkalinity Correction | The specific fraction of buffering capacity contributed by Boron ions rather than the Carbonate system. The math calculates and strips this out to prevent bias. |

# 3. Conceptual Reading Flow

To conceptualize the mathematical sequence from left to right, follow this framework:

1. Isolate Pure Carbonate Base: The device starts with your permanent Day 1 reference value (TA_ref) and subtracts its specific borate component (BA_ref) to ensure it is only analyzing the active carbonate system.

2. Apply Environmental Correction Ratio: It multiplies that clean baseline by a fraction comparing your back-to-back aerated pH probe values. Because both paths were bubbled with identical room air, the volatile room air CO2 variable is completely nullified.

3. Adjust for Current Live State: Finally, the software calculates the specific borate buffering capacity of the active aquarium sample (BA_tank) based on the live temperature probe reading and adds it back to compute the precise Total Alkalinity.