# Corrected DIY Ratiometric Alkalinity Monitor Firmware

Trend-focused ESP32/Arduino C++ framework for a reagentless alkalinity trend monitor using a reference sample, tank sample, fixed 35 ppt salinity assumption, 10-minute aeration cycle, display/logging hooks, and optional controller-readable BNC/DAC trend output.

# What Was Fixed

| Area | Change |
| --- | --- |
| Unit handling | Manual alkalinity is entered as dKH, converted internally to equivalents per liter, used with borate in equivalent units, then converted back to dKH for display. |
| Constants | The code now uses one clearly named 35 ppt trend-mode constant set instead of placeholder helper functions that implied temperature compensation was implemented. |
| Sensor reads | The single dummy pH read was replaced with a timed averaging routine with range checks and basic variance reporting. Sensor functions remain isolated so Atlas EZO or another pH circuit can be dropped in. |
| Fixed-delay read behavior | The aeration assumption remains 10 minutes, but the code now averages pH and temperature after aeration instead of grabbing one value. |
| Fault handling | The firmware now rejects invalid pH, invalid temperature, invalid reference/tank ratio, and invalid final alkalinity values. |
| EEPROM handling | Reference alkalinity storage now uses a small struct with a signature so empty memory is not confused with a valid calibration. |
| Fluidics consistency | The cycle includes an optional double-rinse before the tank test to reduce reference carryover. |
| BNC/controller output | A configurable alkalinity-to-fake-pH mapping and DAC output hook were added, with warnings that real pH ports require isolated millivolt-level hardware, not a direct ESP32 DAC pin. |

# Important Hardware Notes

Do not connect an ESP32 DAC pin directly to an aquarium controller pH/BNC input. A real pH input expects a high-impedance millivolt-scale electrode signal. Use an isolated precision DAC or isolated analog output stage designed to emulate that range.

This code is trend-mode firmware. It assumes salinity is stable at 35 ppt and that 10 minutes of aeration is sufficient. It is not a lab-grade absolute alkalinity instrument.

Before unattended reef use, add level sensing, leak detection, stuck-pump protection, and a watchdog-driven safe shutdown.

# Corrected Firmware

/* Corrected DIY Ratiometric Alkalinity Trend Monitor Firmware Target: ESP32 / Arduino-compatible MCU Purpose: - Reference sample first, tank sample second. - Both are aerated for the same fixed 10-minute period. - pH is averaged after aeration instead of sampled once. - Manual reference alkalinity is entered in dKH. - Internal alkalinity math uses equivalents per liter. - Final result is converted back to dKH for display/logging. Important: - This is trend-mode code, not a lab-grade carbonate chemistry package. - Salinity is assumed stable at 35 ppt. - K values are fixed for 35 ppt / approximately 25 C trend use. - Replace the simulated sensor functions with your Atlas EZO / DS18B20 code. - Do NOT drive a controller pH/BNC input directly from an ESP32 DAC pin. */ #include <EEPROM.h> #include <math.h> // ============================== // USER CONFIGURATION // ============================== #define SIMULATION_MODE true // Set false after real pH/temp sensor functions are implemented. // Fixed trend-mode seawater assumption. const float SALINITY_PPT = 35.0; // Manual reference alkalinity entered from hobby titration test. // Stored in EEPROM after calibration. const uint32_t EEPROM_SIGNATURE = 0xA11C0DE5; const int EEPROM_ADDR = 0; // Unit conversion. // 1 meq/L = 2.8 dKH, so 1 dKH = 0.357142857 meq/L = 0.000357142857 eq/L. const float DKH_TO_EQ_PER_L = 0.000357142857; const float EQ_PER_L_TO_DKH = 2800.0; // Trend-mode constants at approximately 25 C, 35 ppt. // Keep these on the same pH scale as your calibration approach. // For trend-only use, consistency matters more than absolute accuracy. const float K1_35PPT_25C = 1.42e-6; // Approx pK1 ~ 5.85 const float K2_35PPT_25C = 1.08e-9; // Approx pK2 ~ 8.97 const float KB_35PPT_25C = 2.54e-9; // Approx pKB ~ 8.60 // Total boron at 35 ppt, approximate mol/L-style value for trend use. // Borate alkalinity contribution is monovalent, so mol/L ~= eq/L for B(OH)4-. const float TOTAL_BORON_EQ_PER_L = 0.000416; // ============================== // HARDWARE PIN MAPPING // ============================== const int PUMP_REF_FWD = 4; // Draws from reference reservoir. const int PUMP_REF_REV = 5; // Returns to reference reservoir. const int PUMP_TANK = 6; // Draws fresh tank water. const int PUMP_WASTE = 7; // Drains chamber to waste. const int AIR_PUMP = 8; // Aerates sample chamber. // Optional analog trend output hook. // ESP32 DAC pins are usually GPIO25/GPIO26, but do not connect directly to BNC pH input. const int DAC_TREND_OUT = 25; // ============================== // TIMING CONFIGURATION // ============================== const unsigned long FILL_TIME_MS = 15000; const unsigned long AERATE_TIME_MS = 600000; // 10 minutes. const unsigned long DRAIN_TIME_MS = 17000; const unsigned long RINSE_FILL_MS = 10000; const unsigned long RINSE_DRAIN_MS = 10000; const unsigned long SETTLE_TIME_MS = 5000; const unsigned long TEST_INTERVAL_MS = 14400000; // 4 hours. // Averaging after aeration. const unsigned long READ_WINDOW_MS = 60000; // 60 seconds of readings. const unsigned long READ_PERIOD_MS = 2000; // One reading every 2 seconds. // Safety limits. const float MIN_VALID_PH = 6.50; const float MAX_VALID_PH = 9.20; const float MIN_VALID_TEMP_C = 15.0; const float MAX_VALID_TEMP_C = 35.0; const float MIN_VALID_DKH = 3.0; const float MAX_VALID_DKH = 20.0; // Fake-pH trend output mapping for aquarium controller graphing. // Example: 6 dKH -> 7.60 fake pH; 10 dKH -> 8.00 fake pH. const float MAP_DKH_LOW = 6.0; const float MAP_DKH_HIGH = 10.0; const float MAP_FAKE_PH_LOW = 7.60; const float MAP_FAKE_PH_HIGH = 8.00; struct CalibrationData { uint32_t signature; float referenceAlk_dKH; }; struct SampleReading { float pH; float tempC; float pHStdDev; bool ok; }; CalibrationData cal; // ============================== // SETUP / LOOP // ============================== void setup() { Serial.begin(115200); EEPROM.begin(512); pinMode(PUMP_REF_FWD, OUTPUT); pinMode(PUMP_REF_REV, OUTPUT); pinMode(PUMP_TANK, OUTPUT); pinMode(PUMP_WASTE, OUTPUT); pinMode(AIR_PUMP, OUTPUT); pinMode(DAC_TREND_OUT, OUTPUT); ensurePumpsOff(); loadCalibration(); if (!calibrationIsValid()) { Serial.println("SYSTEM HALT: No valid reference alkalinity stored."); Serial.println("Open Serial Monitor and send: CAL 8.20"); waitForCalibrationInput(); } Serial.print("Reference alkalinity anchor loaded: "); Serial.print(cal.referenceAlk_dKH, 2); Serial.println(" dKH"); } void loop() { handleSerialCommands(); Serial.println("\nStarting automated alkalinity trend test..."); SampleReading ref = runReferenceCycle(); if (!ref.ok) { failTest("Reference sample read failed."); delay(TEST_INTERVAL_MS); return; } runFlushCycle(true); // true = double rinse. SampleReading tank = runTankCycle(); if (!tank.ok) { failTest("Tank sample read failed."); delay(TEST_INTERVAL_MS); return; } float tankAlk_dKH = calculateAlkalinity_dKH(ref.pH, tank.pH); if (!isfinite(tankAlk_dKH) ¦¦ tankAlk_dKH < MIN_VALID_DKH ¦¦ tankAlk_dKH > MAX_VALID_DKH) { failTest("Calculated alkalinity outside valid range."); delay(TEST_INTERVAL_MS); return; } Serial.println("---- TEST RESULT ----"); Serial.print("Reference pH: "); Serial.println(ref.pH, 4); Serial.print("Reference pH SD: "); Serial.println(ref.pHStdDev, 5); Serial.print("Tank pH: "); Serial.println(tank.pH, 4); Serial.print("Tank pH SD: "); Serial.println(tank.pHStdDev, 5); Serial.print("Delta pH tank-ref: "); Serial.println(tank.pH - ref.pH, 4); Serial.print("Tank alkalinity trend estimate: "); Serial.print(tankAlk_dKH, 2); Serial.println(" dKH"); updateTrendOutput(tankAlk_dKH); // updateDisplay(tankAlk_dKH, ref, tank); // Add LCD/OLED code here. delay(TEST_INTERVAL_MS); } // ============================== // WORKFLOW FUNCTIONS // ============================== SampleReading runReferenceCycle() { Serial.println("Drawing reference sample..."); runPump(PUMP_REF_FWD, FILL_TIME_MS); Serial.println("Aerating reference sample..."); runPump(AIR_PUMP, AERATE_TIME_MS); delay(SETTLE_TIME_MS); SampleReading reading = readAveragedSample(); Serial.println("Returning reference sample to reservoir..."); runPump(PUMP_REF_REV, DRAIN_TIME_MS); return reading; } void runFlushCycle(bool doubleRinse) { int cycles = doubleRinse ? 2 : 1; for (int i = 0; i < cycles; i++) { Serial.print("Rinse cycle "); Serial.println(i + 1); runPump(PUMP_TANK, RINSE_FILL_MS); runPump(PUMP_WASTE, RINSE_DRAIN_MS); } } SampleReading runTankCycle() { Serial.println("Drawing tank sample..."); runPump(PUMP_TANK, FILL_TIME_MS); Serial.println("Aerating tank sample..."); runPump(AIR_PUMP, AERATE_TIME_MS); delay(SETTLE_TIME_MS); SampleReading reading = readAveragedSample(); Serial.println("Draining tank sample to waste..."); runPump(PUMP_WASTE, DRAIN_TIME_MS); return reading; } void runPump(int pin, unsigned long ms) { digitalWrite(pin, HIGH); delay(ms); digitalWrite(pin, LOW); delay(250); } void ensurePumpsOff() { digitalWrite(PUMP_REF_FWD, LOW); digitalWrite(PUMP_REF_REV, LOW); digitalWrite(PUMP_TANK, LOW); digitalWrite(PUMP_WASTE, LOW); digitalWrite(AIR_PUMP, LOW); } // ============================== // MEASUREMENT FUNCTIONS // ============================== SampleReading readAveragedSample() { SampleReading result; result.pH = NAN; result.tempC = NAN; result.pHStdDev = NAN; result.ok = false; const int maxSamples = READ_WINDOW_MS / READ_PERIOD_MS; float pHValues[40]; float tempSum = 0.0; int n = 0; unsigned long start = millis(); while ((millis() - start) < READ_WINDOW_MS && n < 40) { float pH = read_pH_Probe(); float tempC = read_Temp_C(); if (isfinite(pH) && isfinite(tempC) && pH >= MIN_VALID_PH && pH <= MAX_VALID_PH && tempC >= MIN_VALID_TEMP_C && tempC <= MAX_VALID_TEMP_C) { pHValues[n] = pH; tempSum += tempC; n++; } delay(READ_PERIOD_MS); } if (n < 5) { Serial.println("ERROR: Too few valid pH readings."); return result; } float sum = 0.0; for (int i = 0; i < n; i++) sum += pHValues[i]; float mean = sum / n; float varSum = 0.0; for (int i = 0; i < n; i++) { float d = pHValues[i] - mean; varSum += d * d; } float sd = sqrt(varSum / n); result.pH = mean; result.tempC = tempSum / n; result.pHStdDev = sd; result.ok = true; return result; } // ============================== // ALKALINITY MATH // ============================== float calculateAlkalinity_dKH(float pH_ref, float pH_tank) { float H_ref = pow(10.0, -pH_ref); float H_tank = pow(10.0, -pH_tank); float BA_ref_eqL = borateAlkalinity_eqL(H_ref); float BA_tank_eqL = borateAlkalinity_eqL(H_tank); float TA_ref_eqL = cal.referenceAlk_dKH * DKH_TO_EQ_PER_L; float carbonate_ref_eqL = TA_ref_eqL - BA_ref_eqL; float carbTerm_ref = carbonateSpeciationTerm(H_ref); float carbTerm_tank = carbonateSpeciationTerm(H_tank); if (!isfinite(carbTerm_ref) ¦¦ !isfinite(carbTerm_tank) ¦¦ carbTerm_ref <= 0.0 ¦¦ carbonate_ref_eqL <= 0.0) { return NAN; } float carbonateRatio = carbTerm_tank / carbTerm_ref; float TA_tank_eqL = (carbonate_ref_eqL * carbonateRatio) + BA_tank_eqL; float TA_tank_dKH = TA_tank_eqL * EQ_PER_L_TO_DKH; return TA_tank_dKH; } float carbonateSpeciationTerm(float H) { return (K1_35PPT_25C / H) + ((2.0 * K1_35PPT_25C * K2_35PPT_25C) / (H * H)); } float borateAlkalinity_eqL(float H) { return (TOTAL_BORON_EQ_PER_L * KB_35PPT_25C) / (KB_35PPT_25C + H); } // ============================== // CONTROLLER-READABLE TREND OUTPUT // ============================== void updateTrendOutput(float alk_dKH) { float fakePH = mapFloat(alk_dKH, MAP_DKH_LOW, MAP_DKH_HIGH, MAP_FAKE_PH_LOW, MAP_FAKE_PH_HIGH); fakePH = constrain(fakePH, MAP_FAKE_PH_LOW, MAP_FAKE_PH_HIGH); Serial.print("Mapped fake-pH trend value: "); Serial.println(fakePH, 3); // Placeholder for external isolated DAC stage. // A real pH port expects roughly -414 mV to +414 mV around pH 7 at 25 C. // Do not wire this pin directly to a controller BNC/pH input. int dacValue = (int)round(mapFloat(fakePH, MAP_FAKE_PH_LOW, MAP_FAKE_PH_HIGH, 80.0, 180.0)); #if defined(ESP32) dacWrite(DAC_TREND_OUT, constrain(dacValue, 0, 255)); #else analogWrite(DAC_TREND_OUT, constrain(dacValue, 0, 255)); #endif } float mapFloat(float x, float inMin, float inMax, float outMin, float outMax) { return (x - inMin) * (outMax - outMin) / (inMax - inMin) + outMin; } // ============================== // CALIBRATION / EEPROM // ============================== void loadCalibration() { EEPROM.get(EEPROM_ADDR, cal); } bool calibrationIsValid() { return cal.signature == EEPROM_SIGNATURE && isfinite(cal.referenceAlk_dKH) && cal.referenceAlk_dKH >= MIN_VALID_DKH && cal.referenceAlk_dKH <= MAX_VALID_DKH; } void saveCalibration(float referenceAlk_dKH) { cal.signature = EEPROM_SIGNATURE; cal.referenceAlk_dKH = referenceAlk_dKH; EEPROM.put(EEPROM_ADDR, cal); EEPROM.commit(); Serial.print("Saved reference alkalinity anchor: "); Serial.print(referenceAlk_dKH, 2); Serial.println(" dKH"); } void waitForCalibrationInput() { while (!calibrationIsValid()) { handleSerialCommands(); delay(100); } } void handleSerialCommands() { if (!Serial.available()) return; String line = Serial.readStringUntil('\n'); line.trim(); if (line.startsWith("CAL ")) { float v = line.substring(4).toFloat(); if (v >= MIN_VALID_DKH && v <= MAX_VALID_DKH) { saveCalibration(v); } else { Serial.println("Invalid CAL value. Example: CAL 8.20"); } } else if (line == "STATUS") { Serial.print("Reference alkalinity anchor: "); Serial.print(cal.referenceAlk_dKH, 2); Serial.println(" dKH"); } else if (line == "PUMPS_OFF") { ensurePumpsOff(); Serial.println("All pumps off."); } else { Serial.println("Commands: CAL 8.20, STATUS, PUMPS_OFF"); } } void failTest(const char* message) { ensurePumpsOff(); Serial.print("TEST FAILED: "); Serial.println(message); } // ============================== // SENSOR HOOKS // Replace these with real Atlas EZO-pH and DS18B20 functions. // ============================== float read_pH_Probe() { #if SIMULATION_MODE // Simulated stable reading with tiny pseudo-variation for bench testing. static float base = 8.150; return base + ((millis() % 1000) / 1000.0 - 0.5) * 0.002; #else // TODO: Query Atlas Scientific EZO-pH by I2C/UART and return calibrated pH. // Example structure: // send read command // wait required response time // parse returned pH string // return pH; return NAN; #endif } float read_Temp_C() { #if SIMULATION_MODE return 25.0; #else // TODO: Query DS18B20 or other temperature probe and return Celsius. return NAN; #endif }

# Bench Test Checklist

Compile in SIMULATION_MODE first and confirm Serial output produces a valid trend estimate.

Enter calibration through Serial using CAL 8.20, then power-cycle and confirm EEPROM recall with STATUS.

Run pumps with water only and verify fill, return, rinse, and drain times before connecting to tank water.

Replace read_pH_Probe() and read_Temp_C() with real sensor code before trusting results.

Run three repeated tests on the same water sample. For trend use, aim for roughly +/-0.1 to 0.2 dKH repeatability before installing near the aquarium.

Add leak and level sensors before unattended operation.