DIY Ratiometric Alkalinity Monitor

Microcontroller Firmware (C++)

The following C++ framework is designed for execution on an ESP32 or Arduino-compatible microcontroller. It orchestrates the fluidic routing, timing sequences, and the ratiometric mathematical speciation calculations required to determine aquarium total alkalinity without liquid chemical reagents or acids.

#include <EEPROM.h>
#include <math.h>

// --- HARDWARE PIN MAPPING ---
// Pumps (Driven via MOSFETs or Relays)
const int PUMP_REF_FWD  = 4;  // Draws from Reference Reservoir
const int PUMP_REF_REV  = 5;  // Returns to Reference Reservoir
const int PUMP_TANK     = 6;  // Draws fresh water from Aquarium Sump
const int PUMP_WASTE    = 7;  // Drains measurement chamber to Waste
const int AIR_PUMP      = 8;  // Drives the airstone

// Timings (in milliseconds)
const unsigned long FILL_TIME    = 15000;  // 15 seconds to fill chamber
const unsigned long AERATE_TIME  = 600000; // 10 minutes to reach gas equilibrium
const unsigned long PURGE_TIME   = 10000;  // 10 second flush
const unsigned long TEST_INTERVAL = 14400000; // 4 hours between tests

// --- GLOBAL VARIABLES ---
float TA_ref_anchor = 0.0; // Stored Day 1 manual test value (e.g., 8.2 dKH)
float BT = 0.000416;       // Total Boron at 35 ppt (mol/kg)

void setup() {
  Serial.begin(115200);
  EEPROM.begin(512);

  // Initialize Pump Pins
  pinMode(PUMP_REF_FWD, OUTPUT);
  pinMode(PUMP_REF_REV, OUTPUT);
  pinMode(PUMP_TANK, OUTPUT);
  pinMode(PUMP_WASTE, OUTPUT);
  pinMode(AIR_PUMP, OUTPUT);
  
  ensurePumpsOff();

  // Load the Day 1 Anchor from memory (Assuming stored at address 0)
  EEPROM.get(0, TA_ref_anchor);
  
  // If memory is empty (first boot), prompt user for calibration
  if (isnan(TA_ref_anchor) || TA_ref_anchor <= 0) {
    Serial.println("SYSTEM HALT: No Reference Anchor Found.");
    Serial.println("Please run Day 1 Calibration Protocol.");
    while(1); // Wait for user to input test kit value via Serial/UI
  }
}

void loop() {
  Serial.println("Starting Automated Alkalinity Test...");

  // STEP 1: TEST THE CLOSED REFERENCE
  float pH_ref, temp_ref;
  runReferenceCycle(pH_ref, temp_ref);

  // STEP 2: PURGE CHAMBER (Cross-Contamination Flush)
  runFlushCycle();

  // STEP 3: TEST THE AQUARIUM WATER
  float pH_tank, temp_tank;
  runTankCycle(pH_tank, temp_tank);

  // STEP 4: CALCULATE FINAL ALKALINITY
  float current_tank_alk = calculateAlkalinity(pH_ref, temp_ref, pH_tank, temp_tank);
  
  Serial.print("Current Aquarium Alkalinity (dKH): ");
  Serial.println(current_tank_alk);

  // Sleep until next interval
  delay(TEST_INTERVAL); 
}

// ==========================================
// WORKFLOW FUNCTIONS
// ==========================================

void runReferenceCycle(float &pH_val, float &temp_val) {
  Serial.println("Drawing Reference Sample...");
  digitalWrite(PUMP_REF_FWD, HIGH);
  delay(FILL_TIME);
  digitalWrite(PUMP_REF_FWD, LOW);

  Serial.println("Aerating Reference...");
  digitalWrite(AIR_PUMP, HIGH);
  delay(AERATE_TIME);
  digitalWrite(AIR_PUMP, LOW);

  // Allow water to settle briefly, then read
  delay(2000); 
  pH_val = read_pH_Probe(); 
  temp_val = read_Temp_Probe(); 

  Serial.println("Returning Reference to Reservoir...");
  digitalWrite(PUMP_REF_REV, HIGH);
  delay(FILL_TIME + 2000); // Run slightly longer to ensure lines are clear
  digitalWrite(PUMP_REF_REV, LOW);
}

void runFlushCycle() {
  Serial.println("Flushing Chamber...");
  digitalWrite(PUMP_TANK, HIGH);
  delay(PURGE_TIME);
  digitalWrite(PUMP_TANK, LOW);

  digitalWrite(PUMP_WASTE, HIGH);
  delay(PURGE_TIME);
  digitalWrite(PUMP_WASTE, LOW);
}

void runTankCycle(float &pH_val, float &temp_val) {
  Serial.println("Drawing Tank Sample...");
  digitalWrite(PUMP_TANK, HIGH);
  delay(FILL_TIME);
  digitalWrite(PUMP_TANK, LOW);

  Serial.println("Aerating Tank Sample...");
  digitalWrite(AIR_PUMP, HIGH);
  delay(AERATE_TIME);
  digitalWrite(AIR_PUMP, LOW);

  delay(2000);
  pH_val = read_pH_Probe();
  temp_val = read_Temp_Probe();

  Serial.println("Draining Tank Sample to Waste...");
  digitalWrite(PUMP_WASTE, HIGH);
  delay(FILL_TIME + 2000);
  digitalWrite(PUMP_WASTE, LOW);
}

// ==========================================
// RATIOMETRIC MATH ENGINE
// ==========================================

float calculateAlkalinity(float pH_ref, float temp_ref, float pH_tank, float temp_tank) {
  
  // 1. Convert pH to Hydrogen Ion Concentration
  float H_ref = pow(10, -pH_ref);
  float H_tank = pow(10, -pH_tank);

  // 2. Fetch Thermodynamic Constants (Dependent on temp at 35 ppt)
  // Note: You must implement Millero/Dickson polynomial equations inside these helper functions
  float K1_ref = get_K1(temp_ref);
  float K2_ref = get_K2(temp_ref);
  float KB_ref = get_KB(temp_ref);

  float K1_tank = get_K1(temp_tank);
  float K2_tank = get_K2(temp_tank);
  float KB_tank = get_KB(temp_tank);

  // 3. Calculate Borate Contamination for both states
  float BA_ref = (BT * KB_ref) / (KB_ref + H_ref);
  float BA_tank = (BT * KB_tank) / (KB_tank + H_tank);

  // 4. Run the Master Ratiometric Formula
  // Calculate the Carbonate Speciation terms
  float carb_term_tank = (K1_tank / H_tank) + ((2 * K1_tank * K2_tank) / pow(H_tank, 2));
  float carb_term_ref = (K1_ref / H_ref) + ((2 * K1_ref * K2_ref) / pow(H_ref, 2));
  
  // The Ratio
  float carbonate_ratio = carb_term_tank / carb_term_ref;

  // Final Assembly: (Total_Ref - Borate_Ref) * Ratio + Borate_Tank
  float calculated_TA = (TA_ref_anchor - BA_ref) * carbonate_ratio + BA_tank;

  return calculated_TA;
}

// ==========================================
// SENSOR & MATH HELPER DUMMY FUNCTIONS
// ==========================================

void ensurePumpsOff() {
  digitalWrite(PUMP_REF_FWD, LOW);
  digitalWrite(PUMP_REF_REV, LOW);
  digitalWrite(PUMP_TANK, LOW);
  digitalWrite(PUMP_WASTE, LOW);
  digitalWrite(AIR_PUMP, LOW);
}

float read_pH_Probe() {
  // Insert code here to query your Atlas EZO circuit via I2C/Serial
  return 8.15; // Dummy value
}

float read_Temp_Probe() {
  // Insert code here to query your DS18B20 via OneWire
  // Return value in Kelvin (Celsius + 273.15) for thermodynamic math
  return 298.15; // Dummy value (25C)
}

float get_K1(float temp_Kelvin) {
  // Implement standard seawater K1 polynomial here
  return pow(10, -5.847); // Approximation at 25C, 35ppt
}

float get_K2(float temp_Kelvin) {
  // Implement standard seawater K2 polynomial here
  return pow(10, -8.966); // Approximation at 25C, 35ppt
}

float get_KB(float temp_Kelvin) {
  // Implement standard seawater KB polynomial here
  return pow(10, -8.595); // Approximation at 25C, 35ppt
}